Project 1: Subnetting

Matthew Santiago

University of Maryland Global Campus

CCS 625 Network Engineering

Andrzej (Andrew) Hoppe

October 10, 2024

BallotOnline Class B IP addresses: 172.16.32.0/19 = 8192 IP addresses / IP range: 172.16.32.0-172.16.63.255

Subnet 1: 172.16.32.0/21 = 2048 IP addresses

Network Address: 172.16.32.0

Binary Network Address: 10101100.00010000.00100000.00000000

Subnet Mask: 255.255.248.0

Binary Subnet Mask: 11111111.11111111.11111000.00000000

Broadcast Address: 172.16.39.255

Binary Broadcast Address: 10101100.00010000.00100111.11111111

Usable Host Range: 172.16.32.1 – 172.16.39.254

Number of Usable Hosts: 2046

Subnet 2: 172.16.40.0/21 = 2048 IP addresses

Network Address: 172.16.40.0

Binary Network Address: 10101100.00010000.00101000.00000000

Subnet Mask: 255.255.248.0

Binary Subnet Mask: 11111111.11111111.11111000.00000000

Broadcast Address: 172.16.47.255

Binary Broadcast Address: 10101100.00010000.00101111.11111111

Usable Host Range: 172.16.40.1 – 172.16.47.254

Number of Usable Hosts: 2046

Subnet 3: 172.16.48.0/21 = 2048 IP addresses

Network Address: 172.16.48.0

Binary Network Address: 10101100.00010000.00110000.00000000

Subnet Mask: 255.255.248.0

Binary Subnet Mask: 11111111.11111111.11111000.00000000

Broadcast Address: 172.16.55.255

Binary Broadcast Address: 10101100.00010000.00110111.11111111

Usable Host Range: 172.16.48.1 – 172.16.55.254

Number of Usable Hosts: 2046

Subnet 4: 172.16.56.0/21 = 2048 IP addresses

Network Address: 172.16.56.0

Binary Network Address: 10101100.00010000.00111000.00000000

Subnet Mask: 255.255.248.0

Binary Subnet Mask: 11111111.11111111.11111000.00000000

Broadcast Address: 172.16.63.255

Binary Broadcast Address: 10101100.00010000.00111111.11111111

Usable Host Range: 172.16.56.1 – 172.16.63.254

Number of Usable Hosts: 2046

References:

- Code Beautify. (n.d.). IP to binary converter. Codebeautify.org. https://codebeautify.org/ip-to-binary-converter

- Calculator.net. (n.d.). IP subnet calculator. https://www.calculator.net/ip-subnet-calculator.html